# ECE 6255 — Term Project Report

Digital Processing of Speech Signals — Speech segmentation (voiced / unvoiced / silence)

Georgia Institute of Technology, Spring 2026

Instructor: B.H. Juang

Team: Jaxon Topel, Adrian Cruz, Michael Ritz

# 1.0 Project summary

We segment speech into three classes per short-time frame: silence, voiced, and unvoiced, using short-time energy (STE), zero-crossing rate (ZCR), and autocorrelation-based pitch strength, followed by median filtering and optional higher-level boundary cues.

# 1.1 Notes for consistency

Proofreading: use the heading spelling silence (not “slicence”) in §1.1.1. Use “Michael Ritz” consistently (not “Ruiz”) across report and slides.

# 1.2 Theoretical foundation (equations)

Let x_w[n], n = 0,…,N−1, be a windowed frame (Hamming window). Short-time energy (STE):

E = (1/N) · Σₙ x_w[n]²

Zero-crossing rate (ZCR) for the windowed frame:

ZCR = (1/(2N)) · Σₙ |sgn(x_w[n]) − sgn(x_w[n−1])|

(with sgn(0) treated as +1 to avoid undefined crossings).

Short-time autocorrelation at lag k (unnormalized form):

R(k) = Σₙ x_w[n] · x_w[n+k]

Voicing uses the normalized autocorrelation (coefficient scaling). A prominent peak in the physiological pitch lag range implies periodicity → voiced speech.

# 1.3 / 2.1 Adaptive thresholds

Silence is declared when normalized STE falls below a low percentile of the utterance’s STE distribution. For non-silent frames, a ZCR cutoff is derived from the distribution of ZCR on high-energy (speech-like) frames. Voiced frames require sufficiently high autocorrelation peak height in the F0 lag band and ZCR below that adaptive cutoff; otherwise the frame is unvoiced. Parameters live in params.m (quantiles and voicing_peak_min).

# 3.0 Source code

Public repository: https://github.com/TopelJL/ECE-6255-Digital-Processing-Speech-Signals-Term-Project

# 3.1 Instructions

1. Install MATLAB (R2019b+ recommended).

2. git clone the repository URL above.

3. Open MATLAB and cd to the project root (folder containing main.m).

4. Run: main

5. Select a .wav file when prompted (example_wavs/example1.wav or example2.wav).

6. Inspect the figure and files under results/ (*.png, *.fig).

7. For metrics on the synthetic clip: run evaluate_example2 in MATLAB.

# 3.2 Code running example (video)

[INSERT LINK after upload — e.g. unlisted YouTube URL for the 12-minute team video.]

# 4.0–4.1 Results (fill after running pipeline)

Run evaluate_example2.m and paste accuracy and the 3×3 confusion matrix from results/metrics_example2.txt. Add waveform + segmentation figures from results/ for example1 and example2 (and any team recordings).

Example table placeholder:

| Metric | Value | Notes |  |
| --- | --- | --- | --- |
| Frame accuracy (example2) | Run MATLAB |  |  |
| Confusion matrix | See results/ |  |  |
| Qualitative (example1) | Inspect plots |  |  |

# 5.0 Conclusion

Classical features separate silence, voiced, and unvoiced segments without ML; adaptive thresholds improve robustness across level variations. Future work: spectral flux / centroid, TIMIT alignment, phoneme-level boundaries.

# 6.0 References

L. R. Rabiner and R. W. Schafer, Theory and Applications of Digital Speech Processing, Pearson, 2011.

MathWorks documentation: audioread, hamming, fft, xcorr, medfilt1 — as used in the codebase.

Course materials: ECE 6255, Georgia Tech (Spring 2026), B.H. Juang.